Table

##### Host Country Technical Vehicle Requirements

| Cambodia |  |  |
| --- | --- | --- |
| Vehicle dimensions |  | Maximum |
| Width: All trucks |  | 2.5 m |
| Height: All trucks |  | 4.2 m |
| Length: Rigid motor vehicle |  | 12.2 m |
| Length: Articulated vehicle |  | 16 m |
| Axle loads |  | Maximum |
| Front axles |  |  |
| 1 axle (2 wheels) |  | 6 tons |
| 1 axles (4 wheels) |  | 11 tons |
| Rear axles |  |  |
| 1 axle (4 wheels) |  | 10 tons |
| 2 axles (8 wheels) |  | 19 tons |
| 2 axles with trailer (8 wheels) |  | 19 tons |
| Triple tandem rear axles (12 wheels) |  | 24 tons |
| Triple tandem rear axles on trailer (12 wheels) |  | 24 tons |
| Gross weight of vehicles |  | Maximum |
| All trucks |  |  |
| 2 axles (single front axle, 2 wheels; single rear axle, 4 wheels) |  | 16 tons |
| 3 axles (twin front axles, 4 wheels; single rear axle, 4 wheels) |  | 21 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 8 wheels) |  | 25 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 8 wheels) |  | 30 tons |
| 5 axles (twin front axles, 4 wheels; triple rear axles, 12 wheels) |  | 35 tons |
| 4-axle trailer |  | 35 tons |
| 5-axle trailer |  | 40 tons |
| 4-axle semi-trailer |  | 35 tons |
| 5-axle semi-trailer |  | 40 tons |
| PRC |  |  |
| Vehicle dimensions |  | Maximum |
| Width (combined): All trucks |  | 2.55 m (exception: refrigerated vans allowed up to 2.6 m) |
| Height (combined): All trucks |  | 4 m |
| Length: Goods vehicles |  | 12 m |
| Length: Articulated vehicles |  | 17.1 m (exception: ‘long nose’ vehicles allowed up to 18.1 m) |
| Length: Goods road trucks |  | 20 m (exception: centre axle trailer train of car carrier allowed up to 22 m) |
| Length: Bus (2 axles) |  | 12 m |
| Length: Bus (3 axles) |  | 13.7 m |
| Length: Single articulated bus |  | 18 m |
| Length: Passenger trailer combination |  | 14.5 m |
| Gross weight of vehicles |  | Maximum |
| 2-axle vehicle |  | 18 tons |
| 3-axle vehicle |  | 25 tons |
| Single articulated bus |  | 28 tons |
| 4-axle goods vehicle |  | 31 tons |
| 3-axle vehicle combinations |  | 27 tons |
| 4-axle vehicle combinations |  | 36 tons |
| 5-axle vehicle combinations |  | 43 tons |
| 6-axle vehicle combinations |  | 49 tons |
| Lao PDR |  |  |
| Vehicle dimensions |  | Maximum |
| Width: All trucks |  | 2.5 m |
| Height: All trucks |  | 4.2 m |
| Length: Single Rigid Truck (12 wheels) |  | 12.2 m (exception: less than 12 wheels means special factory set manufacturers?) |
| Length: Semi-trailer Truck |  | 16 m |
| Length: Trailer truck |  | 19 m |
| Axle loads | Maximum | Maximum |
|  | 9.1 Tons per axle roads | 11 Tons per axle roads (only on National Road 3 and National Road 9) |
| Rigid trucks |  |  |
| 2 axles (single front axle, 2 wheels; single rear axle, 2 wheels) | 9.5 tons | 9.5 tons |
| 2 axles (single front axle, 2 wheels; single rear axle, 4 wheels) | 14.1 tons | 15 tons |
| 3 axles (single front axle, 2 wheels; twin rear axle, 4 wheels) | 16 tons | 17 tons |
| 3 axles (single front axle, 2 wheels; twin rear axle, 8 wheels) | 23.2 tons | 25 tons |
| 3 axles (twin front axles, 4 wheels; single rear axle, 2 wheels) | 13.5 tons | 13.5 tons |
| 3 axles (twin front axles, 4 wheels; single rear axle, 4 wheels) | 17.5 tons | 17.5 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 4 wheels) | 20 tons | 22.5 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 8 wheels) | 27.2 tons | 29.5 tons |
| Semi-trailer trucks |  |  |
| 3 axles (10 wheels) | 23.2 tons | 26 tons |
| 4 axles (14 wheels) | 32.3 tons | 36 tons |
| 4 axles (14 wheels, second configuration) | 32.3 tons | 35 tons |
| 5 axles (18 wheels) | 41.4 tons | 45 tons |
| 6 axles (22 wheels) | 49.6 tons | 49.6 tons |
| Trailers |  |  |
| 2 axles (single front axle, 2 wheels; single rear axle, 2 wheels) | 14 tons | 14 tons |
| 2 axles (single front axle, 2 wheels; single rear axle, 4 wheels) | 16.1 tons | 18 tons |
| 2 axles (single front axle, 4 wheels; single rear axle, 4 wheels) | 18.2 tons | 22 tons |
| 3 axles (single front axle, 4 wheels; twin rear axles, 8 wheels) | 27.3 tons | 31 tons |
| Myanmar |  |  |
| Vehicle dimensions |  | Maximum |
| Width |  | 2.5 m |
| Height: Normal vehicle |  | 3.66 m |
| Height: Container carrier |  | 4.6 m |
| Length |  | 12.2 m (This information appears inconsistent with additional information provided elsewhere) |
| Axle load |  | Maximum |
| All vehicles |  | 10 tons |
| Gross weight of vehicles |  | Maximum |
| Vehicle type (general) |  |  |
| 2-axle trucks |  | 16 tons |
| 3-axle trucks |  | 21 tons |
| 4-axle trucks |  | 25 tons |
| Articulated Truck |  | 38 tons |
| Vehicle type (specific) |  |  |
| 3-wheeled motorcycle |  | 0.35 tons |
| Trailer Jeep |  | 0.5 tons |
| Farm Truck (2-cylinder in line) |  | 0.67 tons |
| Hilux / Light Truck (Small) |  | 2.25 tons / 2 tons |
| Light Truck (Middle) (4-wheeled car) |  | 4 tons |
| Farm Truck (4-cyilnder in line) |  | 4.5 tons |
| Light Truck (Big) (6-wheeled car, under 15 feet in length); Mini-busses (24 passengers); Hino (small) (25 passengers) |  | 5.5 tons |
| Light Truck (Big) (over 15 feet in length) |  | 5.5 tons |
| BM Mini-Busses (26-35 passengers) |  | 12 tons |
| Goods big Vehicle (2 axles, 6 wheels) |  | 15 tons |
| Goods big Vehicle (UD Counter 6-wheeled car) |  | 15 tons |
| Heavy Coach (36-45 passengers) |  | 16 tons |
| Heavy Coach (wind Proof) (46 or more passengers) |  | 17 tons |
| Cargo trucks (6 wheels) |  | 16 tons |
| Cargo trucks (3 axles, 8 wheels) |  | 19 tons |
| Cargo trucks (3 axles, 10 wheels) |  | 21 tons |
| Cargo trucks (4 axles, 12 wheels) |  | 25 tons |
| Cargo trucks (Lift axles, 14 wheels, drive with all wheels on road) |  | 29 tons |
| Cargo trucks (4 axles, 14 wheels) |  | 33 tons |
| Cargo trucks (5 axles, 18 wheels) |  | 41 tons |
| Cargo trucks (6 axles, 22 wheels) |  | 48 tons |
| Thailand |  |  |
| Vehicle dimensions |  | Maximum |
| Trucks |  |  |
| Width |  | 2.55 m (2.60 m for truck with refrigerated compartment) |
| Length |  | 12 m |
| Height |  | 4 m (3m if vehicle < 2.30 m wide) |
| Other |  | Front overhang <= 50% of the wheelbase; rear overhang <= 66% of the wheelbase |
| Semi-trailers |  |  |
| Width |  | 2.55 m |
| Length |  | 13.60 m |
| Height |  | 4 m (3 m if vehicle < 2.30 m wide) |
| Other |  | Front overhang <= 50% of the wheelbase; rear overhang <= 40% of the wheelbase; distance from King Pin to rear of trailer <= 12 m |
| Full trailers |  |  |
| Width |  | 2.55 m |
| Length |  | 8 m |
| Height |  | 4 m (3 m if vehicle < 2.30 m wide) |
| Other |  | Front overhang <= 50% of wheelbase; rear overhang <= 50% of wheelbase or <= 66% if compartment is fully enclosed; wheelbase limit of 4.30 m for 2-axle trailers and 3.70 m for 3-axle trailers |
| Buses |  |  |
| Width |  | 2.55 m |
| Length |  | 15 m (if < 12 m, must have 3 axles with steerable rearmost axle) |
| Height |  | 4 m |
| Other |  | Front overhang <= 50% of wheelbase; rear overhang <= 66% of wheelbase |
| Double-decker buses |  |  |
| Width |  | 2.55 m |
| Length |  | 12 m |
| Height |  | 4.30 m |
| Other |  | Front overhang <= 50% of wheelbase; rear overhang <= 66% of wheelbase |
| Rear axle(s) load |  | Maximum |
| Rigid trucks |  |  |
| 2 axles (single front axle, 2 wheels; single rear axle, 2 wheels) |  | 7 tons |
| 2 axles (single front axle, 2 wheels; single rear axle, 4 wheels) |  | 11 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 4 wheels) |  | 13 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 6 wheels) |  | 16.5 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 8 wheels) |  | 20 tons |
| 3 axles (twin front axles, 4 wheels; single rear axle, 4 wheels) |  | 11 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 4 wheels) |  | 13 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 8 wheels) |  | 20 tons |
| Semi-trailers |  |  |
| 1 axle (2 wheels) |  | 7 tons |
| 1 axle (4 wheels) |  | 11 tons |
| 2 axles (4 wheels) |  | 13 tons |
| 2 axles (8 wheels) |  | 20 tons |
| 3 axles (12 wheels) |  | 25.5 tons |
| Full trailers |  |  |
| 2 axles (single front axle, 2 wheels; single rear axle, 2 wheels) |  | 7 tons per axle |
| 2 axles (single front axle, 4 wheels; single rear axle, 4 wheels) |  | 11 tons per axle |
| Gross weight of vehicles |  | Maximum |
| 2 axles (single front axle, 2 wheels; single rear axle, 2 wheels) |  | 9.5 tons |
| 2 axles (single front axle, 2 wheels; single rear axle, 4 wheels) |  | 15 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 4 wheels) |  | 18 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 6 wheels) |  | 21.5 tons |
| 3 axles (single front axle, 2 wheels; twin rear axles, 8 wheels) |  | 25 tons |
| 3 axles (twin front axles, 4 wheels; single rear axle, 4 wheels) |  | 21 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 4 wheels) |  | 23 tons |
| 4 axles (twin front axles, 4 wheels; twin rear axles, 8 wheels) |  | 30 tons |
| Vietnam |  |  |
| Vehicle dimensions |  | Maximum |
| Oversized vehicles |  |  |
| Length |  | 20 m (or > 1.1 times the length of vehicle) |
| Width |  | 2.5 m |
| Height |  | 4.2 m (except container trucks) |
| Out of gauge loads (i.e., inseparable loads) |  |  |
| Length |  | > 20 m |
| Width |  | > 2.5 m |
| Height |  | > 4.2 m (4.35 m for container trucks) |
| Navigation support and escort vehicles required |  |  |
| Length |  | > 20 m |
| Width |  | > 3.5 m |
| Gross weight of vehicles |  | Maximum |
| General vehicles |  |  |
| Single-axle vehicles |  | <= 10 tons |
| Double-axle vehicles (dependent on length between axles) |  |  |
| < 1 m |  | <= 11 tons |
| >= 1 m; < 1.3 m |  | <= 16 tons |
| > 1.3 m |  | <= 18 tons |
| Triple-axle vehicles (dependent on length between centre axle and adjoining axles) |  |  |
| <= 1.3 m |  | <= 21 tons |
| > 1.3 m |  | <= 24 tons |
| Semi-trailer or tractor trucks |  |  |
| 2 axles |  | <= 16 tons |
| 3 axles |  | <= 24 tons |
| 4 axles |  | <= 30 tons |
| >= 5 axles (dependent on distance between first and last axle) |  |  |
| <= 7 m |  | <= 32 tons |
| > 7 m |  | <= 34 tons |
| Truck fleet and trailer trucks |  |  |
| 3 axles |  | <= 26 tons |
| 4 axles |  | <= 34 tons |
| 5 axles (dependent on the distance from the core of the tractor to the core of the first axle of the trailer) |  |  |
| >= 3.2 m; <= 4.5 m |  | <= 38 tons |
| > 4.5 m |  | <= 42 tons |
| 6 axles (dependent on the distance from the core of the tractor to the core of the first axle of the trailer) |  |  |
| >= 3.2 m; <= 4.5 m |  | <= 40 tons (truck compound); and <= 42 tons (rigid truck) |
| > 4.5 m; <= 6.5 m |  | <= 44 tons |
| > 6.5 m |  | <= 48 tons |

•   •   •